Experiment No: - 05

Experiment Name:- DHCP/DNS configuration

Working Procedure:

Step By Step Configuration:
Step 1:

- Click on routerconfigurefast ethernet 1give ip address

- Click on routerconfigurefast ethernet 2give ip address

- Click on routerCLI

- ip dhcp pool p1

- network 192.168.1.0   255.255.255.0

- default-router 192.168.1.2

- exit

- ip dhcp pool p2

- network 192.168.2.0   255.255.255.0

- default-router 192.168.2.2

Step 2:

- Click on DHCPdesktopip address

- Click on DHCPdesktopservicedhcpondefault gateway(router ip attached with dhcp) DNS server(ip address of server) save

- Click on DHCPdesktopservicedhcpchange pool namedefault gateway(router ip attached with dhcp) DNS server(ip address of server) add

Step 3(for all PC’s)

- Click on PCdesktopip configdhcp

Step 4:

- Click on DNSdesktopip config

- Click on DNSdesktopip configdhcp ondefault gateway(router ip attached with dhcp) DNS server(ip address of server)

Step 5:

- Click on Web serverdesktopip config

- Click on Web serverdesktopip configdhcp ondefault gateway(router ip attached with dhcp) DNS server(ip address of server)

Step 6:

- Click on DHCPservicesindex.htmldo changing if you want

- Click on DNS servicesindex.htmldo changing if you want

- Click on Web Server servicesindex.htmldo changing if you want

Step 7:

- Click on DNSservicesDNSon name (www format)Address (ip address of DHCP )

Step 8:

- Click on PCdesktopweb browsergive name or ip address to search

SELF STUDY: DHCP RELAY

Router DHCP configuration: